| Device | Item | Model Unit | GL-250P |
| --- | --- | --- | --- |
| Injection | Screw diameter | mm | 65 |
|  | Screw L/D ratio | L/D | 23 |
|  | Theoretical shot volume | cm3 | 829 |
|  | Injection weight （PET） | g | 900 |
|  | Injection pressure | MPa | 141.6 |
|  | Injection rate | g/s | 200 |
|  | Plastizing capacity | g/s | 38.2 |
|  | Screw speed range | r/min | ０～110 |
| Making-up | Clamping force | kn | 2500 |
|  | Clearance between tie bars | mm | 570×570 |
|  | Opening stroke | mm | 550 |
|  | Max mould height | mm | 600 |
|  | Min mould height | mm | 250 |
|  | Ejector force | kn | 70 |
|  | Ejector stroke | mm | 150 |
| Other | Pump power | kw | 22 |
|  | Heating power | kw | 18.32 |
|  | Oil Tank capacity | l | 500 |
|  | Machine weight | t | 8.5 |
|  | Weight dimension | m | 6.4×1.5×2 |

GL-250P