GL-250

| Device | Item | Model Unit | GL-250 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |
| Injection | Screw diameter | mm | 55 | 60 | 60 |
|  | Screw L/D ratio | L/D | 24 | 21.8 | 20 |
|  | Theoretical shot volume | cm3 | 540 | 653 | 777 |
|  | Injection weight(PS) | g | 491 | 594 | 707 |
|  | Injection pressure | MPa | 239.4 | 137 | 162 |
|  | Injection rate(PS) | g/s | 113 | 137 | 162 |
|  | Plastizing capacity(PS) | g/s | 22.5 | 26.8 | 33 |
|  | Screw speed range | r/min | ０～150 |  |  |
| Making-up | Clamping force | kn | 2500 |  |  |
|  | Clearance between tie bars | mm | 570×570 |  |  |
|  | Opening stroke | mm | 550 |  |  |
|  | Max mould height | mm | 600 |  |  |
|  | Min mould height | mm | 250 |  |  |
| Other | Ejector force | kn | 70 |  |  |
|  | Ejector stroke | mm | 150 |  |  |
|  | Pump power | kw | 22 |  |  |
|  | Heating power | kw | 15.6 |  |  |
|  | Oil tank capacity | l | 500 |  |  |
|  | Machine weight | t | 8.2 |  |  |
|  | Machine dimension | m | 6.2×1.55×2 |  |  |