GL-400

| Device | Item | Model Unit | GL-400 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |
| Injection | Screw diameter | mm | 70 | 75 | 80 |
|  | Screw L/D ratio | L/D | 21.4 | 20 | 18.75 |
|  | Theoretical shot volume | cm3 | 1307 | 1500 | 1708 |
|  | Injection weight(PS) | g | 1112 | 1276 | 1452 |
|  | Injection pressure | MPa | 204 | 177.5 | 156 |
|  | Injection rate(PS) | g/s | 347 | 398 | 453 |
|  | Plastizing capacity(PS) | g/s | 48 | 56.4 | 67 |
|  | Screw speed range | r/min | ０～180 |  |  |
| Making-up | Clamping force | kn | 4000 |  |  |
|  | Clearance between tie bars | mm | 720×720 |  |  |
|  | Opening stroke | mm | 710 |  |  |
|  | Max mould height | mm | 720 |  |  |
|  | Min mould height | mm | 250 |  |  |
| Other | Ejector force | kn | 110 |  |  |
|  | Ejector stroke | mm | 200 |  |  |
|  | Pump power | kw | 37 |  |  |
|  | Heating power | kw | 22.75 |  |  |
|  | Oil tank capacity | l | 800 |  |  |
|  | Machine weight | t | 16 |  |  |
|  | Machine dimension | m | 8.0×1.95×2.25 |  |  |