GL-2000

| Device | Item | Model Unit | GL-2000 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |
| Injection | Screw diameter | mm | 140 | 150 | 160 |
|  | Screw L/D ratio | L/D | 22.8 | 21.3 | 20 |
|  | Theoretical shot volume | cm3 | 10924 | 12540 | 14268 |
|  | Injection weight(PS) | g | 9995 | 11475 | 13055 |
|  | Injection pressure | MPa | 191 | 167 | 147 |
|  | Injection rate(PS) | g/s | 1168 | 1342 | 1527 |
|  | Plastizing capacity(PS) | g/s | 172 | 210 | 256 |
|  | Screw speed range | r/min | ０～65 |  |  |
| Making-up | Clamping force | kn | 20000 |  |  |
|  | Clearance between tie bars | mm | 1700×1600 |  |  |
|  | Opening stroke | mm | 1600 |  |  |
|  | Max mould height | mm | 1600 |  |  |
|  | Min mould height | mm | 850 |  |  |
| Other | Ejector force | kn | 435 |  |  |
|  | Ejector stroke | mm | 400 |  |  |
|  | Pump power | kw | 55+55+45 |  |  |
|  | Heating power | kw | 90 |  |  |
|  | Oil tank capacity | l | 2800 |  |  |
|  | Machine weight | t | 160 |  |  |
|  | Machine dimension | m | 15.4×3.8×4.2 |  |  |